DER26 AMS Firmware Bench Test and Bring-Up Guide

ADBMS6830 SMB chain, ADBMS2950/APM, charger CAN, current/temp/fan checks, and ESP32 WiFi dashboard/logger

| Target firmware | DER26-AMS author-updated v3 / CS default fix |
| --- | --- |
| Primary board | AMS STM32F767 with ADBMS6822 interface |
| SMB chain | ADBMS6830 devices on SMBs, normal string expected on CS_B |
| APM path | ADBMS2950/APM on CS_A; debug-only until bench-proven |
| CAN | Classic CAN 2.0, 250 kbit/s |
| Prepared for | Low-voltage bench bring-up before hardware flash/vehicle testing |

Core rule: Start with BMS_OK inhibited, low voltage only, and a logic analyzer/CAN sniffer connected. Prove observability before trusting any measurement or output.

# 1. Bench Safety and Scope

Scope: This document validates firmware bring-up behavior and observability. It does not prove accumulator safety, ADBMS analog accuracy, charger high-voltage behavior, contactor behavior, or vehicle readiness.

| Do | Reason |
| --- | --- |
| Use a low-voltage bench setup first | Limits stored-energy risk while validating SPI/CAN/CLI behavior. |
| Build first with BMS_OK inhibited | Prevents accidental downstream enable during debug. |
| Use a current-limited LV supply | Catches shorts/miswiring before board damage. |
| Keep APM non-gating | ADBMS2950 CS/SPI/PEC/scaling/shunt polarity are not proven yet. |
| Log every CLI command and result | Turns bring-up into reproducible evidence, not memory. |
| Probe CS/SCK/MOSI/MISO before trusting CLI readings | Software can only interpret what the physical bus actually returns. |

| Do not | Why |
| --- | --- |
| Do not connect tractive HV for first SPI/CAN tests | The first target is communication, not pack operation. |
| Do not release BMS_OK until voltage/current/temp/charger gates are understood | Prevents confusing a firmware bring-up with a safety release. |
| Do not let ESP32 dashboard share ADBMS SPI/isoSPI | Dashboard is passive CAN only. |
| Do not use APM current for safety decisions yet | The path is debug-only until bench-proven. |
| Do not assume ST-Link debugger is available | UART CLI and CAN logs are the primary fallback bring-up path. |

# 2. Equipment and Minimum Setup

| Item | Use | Notes |
| --- | --- | --- |
| Current-limited LV bench supply | Power AMS GLV side | Set conservative current limit before connecting. |
| USB-UART adapter | CLI access | Confirm voltage levels before connecting. |
| Logic analyzer / scope | SPI/CS/isoSPI observation | Minimum channels: CS_A, CS_B, SCK, MOSI, MISO, GND. |
| CAN sniffer | Charger and logger CAN validation | Must support 250 kbit/s and extended IDs. |
| ESP32 + MCP2515 dashboard node | Passive WiFi dashboard/logger | Only on low-voltage CAN bus. |
| Known-good SMB or SMB chain | ADBMS6830 validation | Start with the smallest safe chain the hardware team supports. |
| Charger or charger simulator | Charger CAN bring-up | Use LV/CAN-only mode first if available. |
| DMM | BMS_OK, supply rails, basic continuity | Scope is preferred for transients. |

# 3. Firmware Build and Flash Plan

Use two firmware personalities: one for first hardware bring-up, and one for intentional APM debug.

| Build | Flags | Use |
| --- | --- | --- |
| First SMB bring-up | AMS_HW_BRINGUP=1, AMS_ENABLE_APM_2950_DEBUG=0 | Default. BMS_OK is inhibited and APM is not initialized. |
| APM CLI probing | AMS_HW_BRINGUP=1, AMS_ENABLE_APM_2950_DEBUG=1 | Only after SMB SPI is understood and CS_A routing is intentionally being tested. |
| Normal firmware | AMS_HW_BRINGUP=0 after bench proof | Only after the hardware team accepts the bring-up evidence. |

Expected first CLI banner hints:
Build:hw-bringup
APM2950:0 or 1 depending on build
BMS_OK_inhibit:1
ADBMS6822 SPI6 expected: mode3 CPOL HIGH CPHA 2EDGE

Important: For first flash, do not rely on live debugger breakpoints. If ST-Link debug is still flaky, use UART CLI, CAN sniffer, and logic-analyzer captures.

# 4. Hardware Wiring Reference

## 4.1 AMS ADBMS SPI / isoSPI

| Signal | Expected connection/check | Test command |
| --- | --- | --- |
| SPI6 SCK | Idle high, clocks during command and dummy readback | spi probe / spi sid |
| SPI6 MOSI | MSB-first command bytes and PEC visible | spi probe |
| SPI6 MISO | Not stuck if the chain responds | spi probe / spi sid |
| CS_B | Normal SMB/ADBMS6830 chain chip-select | spi probeb, spi probe, spi sid |
| CS_A | APM/ADBMS2950 path and explicit routing test | spi probea, apm probe |
| ADBMS6822 IP/IM | isoSPI physical pair if accessible | spi sid / spi stat |

SPI mode: Drexel AMS straps ADBMS6822 PHAPOL/PHAPOL2 high, so SPI6 should be mode 3: CPOL HIGH and CPHA 2EDGE. Do not switch to mode 0 just because an ADI eval-board guide says mode 0.

## 4.2 ESP32 WiFi Dashboard / Logger

| ESP32 pin | MCP2515 pin | Notes |
| --- | --- | --- |
| GPIO23 | SI / MOSI | SPI MOSI |
| GPIO19 | SO / MISO | SPI MISO |
| GPIO18 | SCK | SPI clock |
| GPIO5 | CS | SPI chip select |
| GPIO4 | INT | Optional; current driver polls |
| 3V3 | VCC | Use a 3.3 V-compatible module or level shifting |
| GND | GND | Common low-voltage ground reference |
| CAN_H | CAN_H | AMS low-voltage CAN bus |
| CAN_L | CAN_L | AMS low-voltage CAN bus |

| Dashboard item | Value |
| --- | --- |
| CAN bitrate | 250 kbit/s |
| MCP2515 crystal assumption | 8 MHz |
| WiFi SSID | DER26_AMS_DASH |
| WiFi password | der26amslogger |
| Dashboard URL | http://192.168.4.1/ |
| JSON endpoint | /api/state |
| CSV snapshot endpoint | /api/snapshot.csv |

Dashboard boundary: The ESP32 dashboard is passive CAN only. It must not drive BMS_OK, charger enable, AIRs, ADBMS SPI/isoSPI, or any fault latch.

# 5. First Power-Up Checklist

1. Inspect AMS board, SMB connector orientation, low-voltage power, grounds, CAN termination, and UART wiring before applying power.

2. Set bench supply current limit low enough to catch wiring errors.

3. Power only AMS low-voltage electronics. Do not connect tractive HV.

4. Open UART CLI and capture the full session log.

5. Run status and bmsok status before any SPI probing.

6. Verify BMS_OK is inhibited and low.

7. Verify SPI6 reports CPOL HIGH and CPHA 2EDGE.

8. Connect logic analyzer channels before running spi probe/probea/probeb.

status
bmsok status
fault
spi status

| Check | Expected first result |
| --- | --- |
| BMS_OK | 0 / inhibited |
| SPI mode | CPOL:HIGH, CPHA:2EDGE |
| Voltage/temp/current | May be not_ready until real scans complete |
| Hard fault | Expected until required safety data is valid |
| APM | Not initialized unless AMS_ENABLE_APM_2950_DEBUG=1 |

# 6. ADBMS6830 / SMB Chain Bring-Up

## 6.1 CS Routing Proof

This validates the bug-prone part first: CS_A and CS_B must be distinct, and normal SMB traffic must use CS_B.

spi clear
spi enable
spi probea
spi probeb
spi status

| Command | Expected hardware observation | Expected CLI value |
| --- | --- | --- |
| spi probea | Only CS_A pulses | RDCFGA CS_A/stringA probe status printed |
| spi probeb | Only CS_B pulses | RDCFGA CS_B/stringB probe status printed |
| spi probe | Normal probe uses default SMB string; expected CS_B | RDCFGA probe status printed |
| spi status | No extra CS toggles unless command issued | last string/op/status/counters visible |

Acceptance: Do not continue until probea and probeb visibly select different CS pins and normal SMB traffic uses CS_B.

## 6.2 Wake, SID, Status, and Config Readback

spi coldwake
spi probe
spi sid
spi stat
spi cfgchk
spi status

| Good sign | Meaning |
| --- | --- |
| spi probe returns OK or a real HAL error | Firmware transaction path is observable. |
| SID returns stable nonzero serial IDs | Chain is responding and order is stable. |
| RDSTAT fields valid | Status read path works and flags are decoded. |
| cfgchk OK, mismatch masks 0 | Packed config and readback agree. |
| PEC pass masks cover expected ICs | Response framing and PEC are plausible. |
| Command-counter mismatch mask 0 | No obvious missed/interleaved transactions. |

| Bad sign | Likely area | Next action |
| --- | --- | --- |
| RX all 00/all FF | MISO/isoSPI/power/wakeup | Check ADBMS6822 power, isoSPI pair, SMB power, wake timing. |
| PEC fails all ICs | SPI mode/clock/noise/chain count | Confirm mode 3, lower speed, verify SMB count and wiring. |
| CS_B never toggles | GPIO mapping/init | Check main.h CS_B pin and logic analyzer channel. |
| SCK idles low | SPI init wrong | Stop and fix Cube/HAL SPI6 mode. |
| SID order changes | Chain instability/string routing | Repeat sid; inspect SMB harness and isoSPI routing. |
| SPIFLT/sleep persists | Device status/power/wake | Run clrflag, stat again, inspect supply and wake sequence. |

## 6.3 Cell Voltage and Temperature Data

volt
temp
tempsns 0 0
tempsns 0 8
tempsns 0 16
fault
status

Voltage requires a full usable 75-cell topology before BMS_OK can become ready.

Temperature uses 24 sensors per SMB and reads 3 sensors per SMB per ADBMS cycle, so a full temperature sweep takes 8 ADBMS cycles.

After the full sweep is usable, normal partial mux cadence should not create a temperature warning by itself.

Invalid temperature readings near rails should immediately make temp data invalid and fan command should fail to 100%.

tempsns <ic> <sensor> is useful for manually checking mux channel selection and raw thermistor plausibility.

| CLI field | Healthy trend |
| --- | --- |
| voltage usable/updated/stale/PEC | Usable approaches 75; stale and PEC fail remain 0 during stable reads. |
| max/min cell location | Stable and believable for the test pack or simulator. |
| temp usable/stale/invalid | Usable approaches 120 after full sweep; invalid remains 0. |
| temp warning/fanmax/charge_stop | False at room temp with fresh valid sensors. |
| fan percent | 0% below 35 C, ramps 35-50 C, 100% on invalid/fault/fanmax. |

## 6.4 ADBMS Diagnostic Hooks

spi diagclear
spi cfgchk
spi cellst
spi auxdiag
spi status

Open-wire commands should not be spammed during drive-state testing. Use charge/balance/service conditions only.

state charge
spi oweven
spi owodd
spi status
state discharge

| Command | Use | Acceptance |
| --- | --- | --- |
| spi cfgchk | Config readback verification | Status OK, mismatch masks 0. |
| spi cellst | Cell ADC diagnostic hook | Command/status path works with no new link errors. |
| spi auxdiag | GPIO/AUX diagnostic hook | AUX path remains observable; temp path still plausible. |
| spi oweven/owodd | Open-wire command hooks | Only run in charge/balance; inspect deltas per datasheet/wiring. |
| spi diagclear | Reset sticky health counters | Sticky PEC/counter masks clear before repeated test. |

# 7. ADBMS2950 / APM Bring-Up

Safety stance: APM remains debug-only and non-gating. Do not use ADBMS2950 current/voltage for BMS_OK or safety until CS, SPI, PEC, scaling, and shunt polarity are proven on the bench.

Only enable APM probing after SMB/ADBMS6830 communication is understood.

Build with:
AMS_HW_BRINGUP=1
AMS_ENABLE_APM_2950_DEBUG=1

apm status
apm clear
apm enable
apm probe
apm status

| Check | Expected |
| --- | --- |
| CS line | APM probe should pulse CS_A only if APM is wired to CS_A. |
| SPI mode | Same SPI6 mode 3 unless hardware proves otherwise. |
| HAL status | OK is ideal; timeout/error is still useful if counters/previews update. |
| TX/RX preview | TX command visible; RX not all 00/FF once path responds. |
| PEC fail mask | 0 for valid frames; nonzero means framing/PEC/scaling not trustworthy. |
| BMS_OK | Unchanged by APM probe results in current firmware. |

Confirm APM schematic CS path and physical routing before trusting probe results.

Confirm shunt value, current sign, VBAT divider values, ADC code format, and scaling against the ADBMS2950 documentation and board BOM.

Compare APM current against DHAB current only as telemetry at first. A disagreement should be logged, not used as a safety gate yet.

# 8. Current, Temperature, Fan, and BMS_OK Gates

## 8.1 Current Sensor

current
fault
status
bmsok status

| Field | What to check |
| --- | --- |
| current valid | Must be true before BMS_OK is allowed. |
| selected range | 50 A / 800 A range selection should match magnitude. |
| sensor fault reason | Should be OK once ADC pair is fresh and plausible. |
| latched reason | Should remain none unless an overcurrent test intentionally trips. |
| raw ADC and voltages | Useful for DHAB wiring, rail/saturation, and sign debugging. |

## 8.2 Temperature and Fan

temp
tempsns 0 0
status
fault

| Condition | Expected firmware response |
| --- | --- |
| Not ready / stale / invalid temp | temp_fault/read_fault true, fan command 100%, BMS_OK low. |
| Room temp valid | temp_valid true, temp_warning false, fan low/off below 35 C. |
| 45 C hot charge stop | Charge disabled in charge state, warning only. |
| 50 C fan max | Fan 100%, warning only. |
| 60 C sustained | Hard overtemp latch, BMS_OK low. |
| 65 C sustained | Severe diagnostic latch; still BMS_OK low. |

Fan limitation: The firmware verifies commanded fan duty, not physical fan spin, unless the board has tach/current feedback wired and implemented.

## 8.3 BMS_OK Release Procedure

bmsok status
status
fault
volt
current
temp
charger

Only release BMS_OK after every ready gate is understood. In hardware bring-up firmware, release is explicit:

bmsok release
bmsok status
bmsok inhibit

| Ready gate | Required state |
| --- | --- |
| Voltage | valid true, fault false, full usable cell set. |
| Current | valid true, current fault false. |
| Temperature | valid true, temp fault false; in charge state temp_charge_stop must be false. |
| Charger | charger fault false when in charge workflow. |
| ADBMS diagnostics | diag fault false after low-rate status/config checks. |
| Heartbeat | task heartbeat fault false. |
| Manual inhibit | released only after bench operator decision. |

# 9. Charger CAN Bench Test

Low voltage first: Validate charger CAN command/status and disable byte on a sniffer before connecting charger HV output to anything meaningful.

1. Connect AMS CAN to a CAN sniffer at 250 kbit/s.

2. Keep charger HV output disabled/disconnected for first pass.

3. Enter charge state from CLI.

4. Confirm AMS transmits extended ID 0x1806E5F4 once per second.

5. Confirm command payload matches 312.0 V, 10.0 A, enable byte when allowed.

6. Inject inhibit/fault conditions and confirm byte 4 changes to 01.

7. Confirm charger status frame 0x18FF50E5 is received and parsed if real charger is connected.

state charge
charger
bmsok status
# Sniffer expected AMS command when allowed:
# ID 0x1806E5F4, data 0C 30 00 64 00 00 00 00
bmsok inhibit
charger
state discharge

| Frame | ID | Expected |
| --- | --- | --- |
| AMS to charger | 0x1806E5F4 extended | 0C 30 00 64 00 00 00 00 when charge allowed. |
| Charger to AMS | 0x18FF50E5 extended | Read voltage/current and status flags update in charger CLI. |
| Disable test | 0x1806E5F4 byte 4 | 01 when inhibited/faulted/charge stop requires charger off. |

| Symptom | Likely cause | Next check |
| --- | --- | --- |
| No 0x1806E5F4 | Not in charge state / CAN TX issue | state charge, CAN task, hcan, bus wiring. |
| Frame present, no ACK | Wiring/termination/baud/charger config | CAN_H/L polarity, 250 kbit/s, termination. |
| Byte 4 stuck 01 | AMS intentionally disabling | charger disable_mask, BMS_OK, temp/voltage/current validity. |
| rx_comm true | No charger status within 5 s | Real charger status ID/config or bus path. |
| tx_fail true | AMS could not queue command | CAN peripheral/mailboxes/bus fault. |
| flags nonzero | Charger reports internal fault | Decode raw flags and charger manual. |

# 10. ESP32 WiFi Dashboard / Logger Bring-Up

1. Build and flash Tools/esp32_ams_dashboard_logger with ESP-IDF.

2. Confirm the MCP2515 module is 3.3 V-compatible or properly level shifted.

3. Confirm MCP2515 crystal is 8 MHz; update CNF timing if not.

4. Connect CAN_H, CAN_L, and low-voltage ground only. Do not connect to ADBMS SPI or isoSPI.

5. Boot AMS and confirm CAN is active at 250 kbit/s.

6. Connect laptop/phone to DER26_AMS_DASH using password der26amslogger.

7. Open http://192.168.4.1/ and /api/state.

8. Confirm stale:false, increasing rx_frames, heartbeat version 1, and plausible pack/current/temp fields.

9. Use the dashboard CSV download for a quick browser-side log. Use raw CAN logs for serious post-run analysis.

cd Tools/esp32_ams_dashboard_logger
idf.py set-target esp32
idf.py build
idf.py -p <PORT> flash monitor

ESP-IDF CMake note: The MCP2515 component should declare the SPI dependency with `REQUIRES esp_driver_spi` in `idf_component_register(...)`. If `idf.py build` fails around SPI driver visibility, check for an accidental `PRIV_REQUIRES esp_driver_spi` entry and use the public `REQUIRES` form.

Expected ESP32 monitor:
MCP2515 ready: 250 kbit/s @ 8 MHz
WiFi AP started: SSID=DER26_AMS_DASH password=der26amslogger
Open http://192.168.4.1/
AMS dashboard/logger running

| Dashboard problem | Likely cause | Fix |
| --- | --- | --- |
| Page loads but STALE | No valid AMS CAN frames | Check CAN wiring, bitrate, MCP2515 crystal, AMS CAN task. |
| No WiFi AP | ESP32 firmware/power | Check serial monitor and flash target. |
| MCP2515 init failed | SPI wiring/module power/crystal | Check GPIO23/19/18/5, 3V3, GND, module oscillator. |
| Decoded nonsense | Contract mismatch or byte order issue | Compare raw frames to AMS_LOGGER_CAN_CONTRACT. |
| Only charger frames visible | Logger filters/AMS state issue | Check 0x690..0x6A5 frames on sniffer. |

# 11. Suggested Full Bench Run Sequence

| Phase | Commands / action | Exit criteria |
| --- | --- | --- |
| 0. Static setup | Power off inspection, continuity, supply current limit | No obvious shorts or wrong connectors. |
| 1. Boot | status, bmsok status, fault | CLI alive, BMS_OK inhibited, SPI mode correct. |
| 2. CS routing | spi probea, spi probeb, spi probe | CS_A/CS_B distinct; normal SMB path on CS_B. |
| 3. SMB identity | spi coldwake, spi sid, spi stat | Stable SIDs/status, no all-FF/all-00 RX. |
| 4. SMB data | volt, temp, tempsns | Usable/stale/PEC/invalid counts make sense. |
| 5. Diagnostics | spi cfgchk, cellst, auxdiag, open-wire in charge/balance only | No unexpected diagnostic fault; open-wire only run safely. |
| 6. Current | current | Current valid and sign/range are understood. |
| 7. Charger CAN | state charge, charger, CAN sniffer | 0x1806E5F4 payload correct; disable byte behavior proven. |
| 8. Dashboard | Flash ESP32, open web UI | stale:false and decoded data plausible. |
| 9. BMS_OK | bmsok status, release only if ready gates pass | BMS_OK high only when gates truly pass; inhibit returns low. |
| 10. Log evidence | Save CLI log, Saleae capture, CAN trace, dashboard CSV | Enough data exists to debug failures later. |

# 12. Evidence to Save

| Artifact | Filename suggestion | Must include |
| --- | --- | --- |
| UART CLI log | ams_cli_bringup_YYYYMMDD.txt | All commands, outputs, build banner. |
| Logic analyzer capture | adbms_spi_cs_routing.sal | CS_A, CS_B, SCK, MOSI, MISO for probea/probeb/probe/sid. |
| CAN trace | charger_logger_can_YYYYMMDD.csv | 0x1806E5F4, 0x18FF50E5, 0x690..0x6A5. |
| Dashboard CSV | der26_ams_dashboard_log.csv | At least one stable decoded sample. |
| Photos | bench_wiring_overview.jpg | AMS, SMB, CAN, ESP32 wiring visible. |
| Sign-off notes | bringup_notes_YYYYMMDD.md | Known failures, accepted assumptions, next actions. |

# 13. Bench Sign-Off Checklist

| Done | Check | Notes |
| --- | --- | --- |
| ☐ | AMS boots on LV supply without overcurrent. |  |
| ☐ | UART CLI works without debugger. |  |
| ☐ | BMS_OK inhibited at boot and can be inhibited again after release. |  |
| ☐ | SPI6 mode confirmed as CPOL HIGH / CPHA 2EDGE. |  |
| ☐ | spi probea pulses only CS_A. |  |
| ☐ | spi probeb pulses only CS_B. |  |
| ☐ | Normal SMB probe/SID/status use CS_B. |  |
| ☐ | ADBMS6830 SID/status/config readback are stable. |  |
| ☐ | Voltage full usable count reaches expected topology. |  |
| ☐ | Temperature full usable count reaches expected topology after mux sweep. |  |
| ☐ | Fan commands 100% on invalid/stale/not-ready temp path. |  |
| ☐ | Current sensor reads fresh/plausible values and invalid path drops BMS_OK. |  |
| ☐ | APM probe is debug-only and does not affect BMS_OK. |  |
| ☐ | Charger command frame 0x1806E5F4 is present and byte 4 behavior is proven. |  |
| ☐ | ESP32 dashboard shows stale:false on live AMS CAN frames. |  |
| ☐ | All captures/logs saved. |  |

# 14. Known Open Questions

| Question | Why it matters |
| --- | --- |
| ST-Link debug failure mode | Changes whether bring-up relies fully on UART/CAN logs or can use live breakpoints. |
| Exact ADBMS2950/APM scaling and shunt polarity | Needed before APM telemetry can be trusted. |
| CM200DX/charger configuration | CAN IDs, bitrate, voltage/current limits, and byte-4 polarity must match real charger setup. |
| Fan tach/current feedback availability | Without feedback, firmware only proves fan command, not physical airflow. |
| Open-wire safe-state policy | Open-wire testing should remain charge/service/balance only unless hardware team approves otherwise. |